基本資料

- 姓名：王豪逸 - 標題：B024020014_Assignment 4(基本)

作業目的與設計

- 目標：模擬垃圾車 - 電路設計說明(總分1分)： 車子使用的GPIO： 左輪[前(6),後(13)],右輪[前(19),後(26)] 方向燈的GPIO: 左(20)，右(21) 蜂鳴器(18)  電路設計照片(總分1分)： - 程式邏輯設計(總分1分)： Socket server基本上就要有兩種thread，等待用戶連接的跟用戶連接後處理資料的thread。 SocketThread: 等待使用者連接的thread，會卡在accept。使用者連接後會產生ConnectThread。 ConnectThread: 處理使用者資料的thread。會生成控制車子的物件並根據使用者的輸入來使用物件的函式。 車子的部分有音樂跟方向燈這兩個跟時間有關係的動作。我分別弄成兩個thread class AutoCar: 車子的主物件能夠使用已經寫好的函式來控制他，本身也是方向燈閃爍的thread。也有個音樂盒LittltStar來撥放聲音。 LittltStar: 播放小星星的thread，有簡單的函式能夠讓他執行與暫停撥放。 另外傳送的指令讓他有意義並且配合另外製作的簡單APP改成: 前進: up, 後退: down, 左轉: left, 右轉: right, 停止: stop - 影片URL：https://youtu.be/wobk3mB69VY

心得(總分1分)

- 遇到哪些困難： - 車子的馬達一直無法轉動。 - 一直無法使用CTRL+C完整的中止這個程式 - 如何解決： - 做了很多交叉的測試，再找看看是不是L298N壞掉。但是測試了一陣子後發現有些時候一邊能動又有些時候是另一邊。也不是螺絲那部分的接觸不良。最後發現杜邦線接L298N那邊很鬆常常沒有接觸好，拿張紙卡處那些線這個問題就解決了。 - 試過了try except也試過了擷取signal好不容易在window能夠運作了，但是換到樹梅派也是會出問題。社團上有人有提供好的處理方法是thread上處理不當。 這份作業在socket的部分如果不用thread處理的話這程式很容易出問題，但是很多人為了作業要求或是要簡潔程式碼，只用少少的一兩個甚至不想用，這樣有意義嗎。另外也讓我有點好奇，為甚麼要簡潔這些線路。使用的線材多一點但是看起來清楚明瞭不是很好嗎。

程式碼

程式碼： ====基本題===(總分1分) import socket import threading import signal import sys import time import RPi.GPIO as GPIO def main(): GPIO.setmode(GPIO.BCM) sockThread = SocketThread() sockThread.start() try: while(True): time.sleep(10) except KeyboardInterrupt: sockThread.stop() exit() class SocketThread(threading.Thread): def __init__(self): threading.Thread.__init__(self) self.ip = "192.168.2.4"#socket.gethostbyname(socket.gethostname()) self.port = 8089 self.threads = [] def run(self): self.sock = socket.socket(socket.AF_INET, socket.SOCK_STREAM) self.sock.bind( (self.ip, self.port) ) self.sock.listen(5) self.running = True print('Server start at: %s:%s' %(self.ip, self.port)) while self.running: try: conn, addr = self.sock.accept() print ('Connected by ', addr) ct = ConnectThread(conn, addr) self.threads.append(ct) ct.start() except socket.error: pass for ct in self.threads: ct.stop() print("Sock stop") def stop(self): self.running = False for ct in self.threads: ct.stop() self.sock.close() class ConnectThread (threading.Thread): def __init__(self, conn, addr): threading.Thread.__init__(self) self.conn = conn self.addr = addr self.car = AutoCar() self.car.start() def run(self): self.running = True try: self.conn.send("Hello\n".encode()) while self.running: data = self.conn.recv(1024).decode() print (data) self.car.move(data) self.conn.send("server received you message.\n".encode()) except socket.error: print("Disconnect", self.addr) self.stop() def stop(self): self.running = False #self.conn.close() self.car.stop() class AutoCar (threading.Thread): def __init__(self): threading.Thread.__init__(self) self.musicBox = LittltStar() self.musicBox.start() self.carGPIO = [6,13,19,26] self.leftLedGPIO = 20 self.rightLedGPIO = 21 self.leftLed = False self.rightLed = False self.flash = True self.running = False for i in self.carGPIO: GPIO.setup(i, GPIO.OUT) GPIO.output(i, False) GPIO.setup(self.leftLedGPIO, GPIO.OUT) GPIO.setup(self.rightLedGPIO, GPIO.OUT) GPIO.output(self.leftLedGPIO, False) GPIO.output(self.rightLedGPIO, False) def run(self): self.running = True while self.running : GPIO.output(self.leftLedGPIO, self.leftLed) GPIO.output(self.rightLedGPIO, self.rightLed) time.sleep(0.5) if(self.flash): GPIO.output(self.leftLedGPIO, False) GPIO.output(self.rightLedGPIO, False) time.sleep(0.5) def move(self, way): if(way == "right"): self.leftLed = False self.rightLed = True self.flash = True self.musicBox.play() self.setWheel([True, False, False, False]) elif (way == "left"): self.leftLed = True self.rightLed = False self.flash = True self.musicBox.play() self.setWheel([False, False, True, False]) elif (way == "up"): self.leftLed = False self.rightLed = False self.flash = True self.musicBox.play() self.setWheel([True, False, True, False]) elif (way == "down"): self.leftLed = True self.rightLed = True self.flash = True self.musicBox.play() self.setWheel([False, True, False, True]) elif (way == "stop"): self.leftLed = True self.rightLed = True self.flash = False self.musicBox.mute() self.setWheel([False, False, False, False]) def setWheel(self, mode): for i in range(0, len(self.carGPIO)): GPIO.output(self.carGPIO[i], mode[i]) def stop(self): self.running = False self.musicBox.stop() class LittltStar (threading.Thread): def __init__(self): threading.Thread.__init__(self) self.tone = [0, 262, 294, 330, 349, 392, 440] self.chord = [1,1,5,5,6,6,5,4,4,3,3,2,2,1, 5,5,4,4,3,3,2,5,5,4,4,3,3,2, 1,1,5,5,6,6,5,4,4,3,3,2,2,1] self.buzzGPIO = 18 GPIO.setup(self.buzzGPIO, GPIO.OUT) self.buzz = GPIO.PWM(self.buzzGPIO, 440) self.buzz.start(60) self.playMusic = False self.running = False def run(self): temple = 0 self.running = True while self.running : if(self.playMusic): self.buzz.ChangeFrequency(self.tone[self.chord[temple]]) self.buzz.ChangeDutyCycle(60) temple = (temple+1)%len(self.chord) if temple % 7 == 0: time.sleep(0.9) else: time.sleep(0.4) else: time.sleep(0.4) self.buzz.ChangeDutyCycle(0) time.sleep(0.1) def play(self): self.playMusic = True def mute(self): self.playMusic = False def stop(self): self.running = False main()